OxySync Device — Encryption Setup Instructions

Date: July 31, 2026

Device: ESP32 DevKit V1 (NEW TEST UNIT ONLY)

────────────────────────────────────────────────────────────────────────

⚠️  CRITICAL WARNING — READ BEFORE STARTING

This process permanently modifies the device hardware (eFuse registers).

It CANNOT be undone — ever.

Do this ONLY on the new test unit — NOT on any unit currently at the client site.

If the process is interrupted midway, the device may become permanently unusable.

Do NOT disconnect the USB cable during any step.

────────────────────────────────────────────────────────────────────────

## WHAT YOU RECEIVE FROM GearUpCode

Create a folder on your Desktop called OxySync_Encryption and place all these files in it:

oxysync_production.ino.bootloader.signed.bin

oxysync_production.ino.signed.bin

oxysync_production.ino.partitions.bin

boot_app0.bin

oxysync_public_key.pem

STEP 1 — Install Python (skip if already installed)

Go to https://www.python.org/downloads/ and download the latest Python for Windows.

Important: During installation, tick the box "Add Python to PATH".

Open Command Prompt and verify:

python --version

You should see a version number like Python 3.x.x

STEP 2 — Install esptool

Open Command Prompt and run:

pip install esptool

Wait for it to finish. Then verify:

python -m esptool version

You should see esptool.py v5.x.x or similar.

STEP 3 — Connect the device and find the COM port

1. Connect the ESP32 test unit to your PC via USB.

2. Open Device Manager (right-click Start → Device Manager).

3. Look under Ports (COM & LPT).

4. Note the COM port number — e.g. COM3 or COM6.

You will replace COMX in all commands below with your actual COM port number.

STEP 4 — Navigate to the files folder

Open Command Prompt and run:

cd C:\Users\%USERNAME%\Desktop\OxySync_Encryption

STEP 5 — Burn the Secure Boot public key into eFuse  ⚠️ PERMANENT

Run the command below. Replace COMX with your COM port number.

python -m espefuse --port COMX burn_key BLOCK2 oxysync_public_key.pem SECURE_BOOT_V2_RSA_3072

The tool will ask you to type BURN to confirm. Type it exactly and press Enter.

STEP 6 — Enable Secure Boot V2  ⚠️ PERMANENT

Run:

python -m espefuse --port COMX burn_efuse ABS_DONE_1

Type BURN to confirm when prompted.

STEP 7 — Flash the signed firmware

Run this single command (copy and paste the entire line):

python -m esptool --port COMX --baud 460800 write_flash 0x1000 oxysync_production.ino.bootloader.signed.bin 0x8000 oxysync_production.ino.partitions.bin 0xe000 boot_app0.bin 0x10000 oxysync_production.ino.signed.bin

Wait for it to complete. You should see: Hash of data verified. / Hard resetting via RTS pin...

STEP 8 — Enable Flash Encryption  ⚠️ PERMANENT

The device will auto-encrypt its flash on next boot.

python -m espefuse --port COMX burn_efuse FLASH_CRYPT_CNT

Type BURN to confirm.

STEP 9 — Reboot and verify

1. Unplug and replug the USB cable (or press the EN/Reset button).

2. The device will take slightly longer than usual on first boot — this is normal (it is encrypting the flash).

3. On your phone or laptop, connect to WiFi: O2_Controller  (password: techonly123)

4. Open browser and go to: 192.168.4.1

5. Login: username tech / password oxygen

6. Confirm the dashboard loads and sensor readings appear.

STEP 10 — Report back to us

Send us a message confirming:

☐  Dashboard loaded successfully

☐  Sensor reading is showing

☐  Device WiFi is working

☐  Any errors or unexpected behaviour (if any)

────────────────────────────────────────────────────────────────────────

IMPORTANT — Future Firmware Updates

From this point on, only signed firmware from GC can be flashed to this device. Any unsigned firmware will be rejected and the device will not boot. Always contact GC for any future firmware updates.

OxySync Encryption Setup — GC — July 31, 2026